Instruction Manual

----- CSF JET Multi Megasonic Generator -----

LCD Panel User Operation Manual / v4.0-EN / 2026-05-19

Actual LCD operation panel

# < OPERATING SUMMARY >

| Item | Operation |
| --- | --- |
| Run/Stop | Press START/STOP on the SELECT screen |
| Move item | Short press MODE: MODE -> FREQ CH -> 8 POWER |
| Enter setting | Long press MODE on the SELECT screen |
| Change value | UP/DOWN; long press repeats quickly |
| Save | Short press SET; two beeps indicate success |
| Return to select screen | Long press SET for about 2 seconds in setting screens |
| Auto Tuning | Long press START/STOP for about 3 seconds in FREQ SET |
| Clear alarm | Remove cause, then press SET on the alarm screen |

# 1. Specification

| No. | Item | Specification |
| --- | --- | --- |
| 1 | Model name | CSF JET Multi Megasonic Generator |
| 2 | Display | LCD1602, 2 lines x 16 characters |
| 3 | Buttons | START/STOP, MODE, UP, DOWN, SET |
| 4 | Indicators | NORMAL, H/L SET, 8 POWER, REMOTE, EXT, TX/RX LEDs |
| 5 | Operating frequency | 10CH, 400kHz to 2200kHz |
| 6 | Frequency setting | Manual setting and Auto Tuning |
| 7 | LC BIT | 16 steps, BIT 0000 to 1111 |
| 8 | Output setting | 8 POWER profiles, LOW/HIGH/DEF |
| 9 | Output range | 0.20W to 5.00W |
| 10 | Control connector | 25-pin D-SUB |
| 11 | Communication | RS-485 Half-Duplex, Modbus RTU |
| 12 | Setting storage | Internal FLASH, retained after power-off |

# 2. Caution on Product Use

## 2.1 Caution on Installation

Install the generator on a stable support where vibration does not affect the unit.

Avoid acidic/alkaline gas, conductive dust, oil mist and high-temperature environments.

Secure ventilation space around the unit.

Connect protective earth before applying power.

Use the assigned nozzle/transducer and cable.

Do not change output cable type or length without confirmation.

Turn off power before wiring work.

## 2.2 Caution on Use

Do not operate while the output connector is disconnected.

If SENSOR is open, oscillation is inhibited or stopped.

Do not repeatedly restart after an alarm without checking the cause.

After installation or nozzle replacement, start at low output and increase gradually.

Keep sufficient interval between REMOTE ON and OFF.

For communication operation, match address, baudrate and parity with the master.

# 3. Name and Function of Each Part

LCD screen examples

| Part | Function |
| --- | --- |
| LCD | Displays output, frequency, operation mode, setting menu and alarms |
| NORMAL LED | Indicates NORMAL operation mode |
| H/L SET LED | Indicates high/low alarm setting related status |
| 8 POWER LED | Indicates 8 POWER selection/setting |
| REMOTE LED | Indicates REMOTE mode |
| EXT LED | Indicates EXT communication mode |
| TX/RX LED | Indicates RS-485 transmit/receive activity |
| START/STOP | Starts/stops oscillation; long press in FREQ SET starts Auto Tuning |
| MODE | Moves selected item/edit field; long press enters setting or returns to upper menu |
| UP/DOWN | Increases/decreases values; long press repeats quickly |
| SET | Saves/confirms values; long press returns to SELECT screen in setting screens |

# 4. Connecting Method

Rear connection overview

## 4.1 25-pin D-SUB Control Connector

25-pin D-SUB connection

| Pin | Signal | Type | Description |
| --- | --- | --- | --- |
| 1 | LOW ALARM A | Relay contact | SHORT to pin 14 when Err1 occurs |
| 14 | LOW ALARM B | Relay contact | Err1 LOW ALARM |
| 2 | GO A | Relay contact | SHORT to pin 15 in normal output range |
| 15 | GO B | Relay contact | Normal output range |
| 3 | HIGH ALARM A | Relay contact | SHORT to pin 16 when Err2 occurs |
| 16 | HIGH ALARM B | Relay contact | Err2 HIGH ALARM |
| 6 | TRANSDUCER ALARM A | Relay contact | SHORT to pin 19 when Err5 occurs |
| 19 | TRANSDUCER ALARM B | Relay contact | Transducer/load alarm |
| 7 | OPERATE A | Fail-safe relay | SHORT to pin 20 in normal state; OPEN at alarm/power-off |
| 20 | OPERATE B | Fail-safe relay | Integrated operation/alarm contact |
| 9 | COM | Input common | Common for REMOTE/BCD inputs |
| 22 | REMOTE | Contact input | SHORT to pin 9 = RUN, OPEN = STOP |
| 11 | BCD1 | Contact input | 8 POWER external select bit1 |
| 23 | BCD2 | Contact input | 8 POWER external select bit2 |
| 10 | BCD3 | Contact input | 8 POWER external select bit3 |
| 13 | SENSOR A | Safety input | SHORT to pin 25 is normal |
| 25 | SENSOR B | Safety input | OPEN causes Err7/Err8 |
| 4,5,8,12,17,18,21,24 | N.C. | Reserved | Do not use |
| SHIELD | Connector shell | Shield | Connector body shield/ground |

## 4.2 8 POWER External Selection

| BCD3 | BCD2 | BCD1 | Selected POWER |
| --- | --- | --- | --- |
| 0 | 0 | 0 | PWR1 |
| 0 | 0 | 1 | PWR2 |
| 0 | 1 | 0 | PWR3 |
| 0 | 1 | 1 | PWR4 |
| 1 | 0 | 0 | PWR5 |
| 1 | 0 | 1 | PWR6 |
| 1 | 1 | 0 | PWR7 |
| 1 | 1 | 1 | PWR8 |

# 5. Operation Ready

Confirm DI water or process liquid is supplied to the nozzle when required.

Check output cable and 25-pin D-SUB wiring.

Confirm SENSOR is in normal SHORT state.

Turn on power and check LCD/LED indication.

Start the first operation at low output.

# 6. Operating Method

LCD menu setting flow

## 6.1 FREQ Channel

| Channel | Default Frequency | Description |
| --- | --- | --- |
| CH01 | 400kHz | Frequency profile 1 |
| CH02 | 600kHz | Frequency profile 2 |
| CH03 | 800kHz | Frequency profile 3 |
| CH04 | 1000kHz | Frequency profile 4 |
| CH05 | 1200kHz | Frequency profile 5 |
| CH06 | 1400kHz | Frequency profile 6 |
| CH07 | 1600kHz | Frequency profile 7 |
| CH08 | 1800kHz | Frequency profile 8 |
| CH09 | 2000kHz | Frequency profile 9 |
| CH10 | 2200kHz | Frequency profile 10 |

## 6.2 POWER SET

| Field | Meaning | Range |
| --- | --- | --- |
| CH | POWER profile number | PWR1~PWR8 |
| LOW | Low alarm threshold | 0.10W~0.95W |
| HIGH | High alarm threshold | 0.20W~5.00W |
| DEF | Default output | 0.20W~5.00W |

## 6.3 EXT:RS485

| Field | Setting | Description |
| --- | --- | --- |
| BAUD | 9600/19200/38400/115200 | Communication speed |
| ADDR | 1~247 | Slave address |
| TERM | OFF/ON | Termination setting record; hardware control reserved |
| PARITY | NONE-8-1/EVEN-8-1 | Panel parity setting |

MODE long -> SETTING MODE -> select item -> SET enter -> MODE next field -> UP/DOWN change -> SET save

START/STOP for about 3 seconds in FREQ SET: Auto Tuning

Save success: 2 beeps, save failure: 3 beeps

# 7. ERROR Display

| Code | Contents | Condition | Output | Action |
| --- | --- | --- | --- | --- |
| Err1 | LOW ALARM | Output remains below LOW setting | Stop | Check load/cable/LOW setting, then press SET |
| Err2 | HIGH ALARM | Output remains above HIGH setting | Stop | Check output setting/load/HIGH setting, then press SET |
| Err5 | TRANSDUCER ALARM | Transducer or load abnormal | Stop | Check nozzle/transducer/cable |
| Err6 | SETTING ERROR | Setting outside allowed range | Conditional | Enter a value within the allowed range |
| Err7 | SENSOR OFF | SENSOR open while stopped | Output inhibited | Power OFF -> check SENSOR short -> power ON |
| Err8 | SENSOR RUN | SENSOR open while running | Stop | Power OFF -> check SENSOR short -> power ON |

# 8. Communication Specification

| Item | Specification |
| --- | --- |
| Interface | RS-485 Half-Duplex |
| Protocol | Modbus RTU |
| Default setting | ADDR 1, 9600bps, NONE-8-1 |
| Supported baudrates | 9600/19200/38400/115200 |
| Setting menu | SETTING MODE -> 3.EXT:RS485 |
| Function codes | 0x03 / 0x06 / 0x10 |

| Address | R/W | Name | Value |
| --- | --- | --- | --- |
| 0x0000 | R/W | RUN control | 0=STOP, 1=RUN |
| 0x0001 | R/W | Frequency | x0.1kHz |
| 0x0002 | R/W | Duty | x0.1% |
| 0x0006 | R/W | Operation mode | 0=NORMAL,1=REMOTE,2=EXT |
| 0x0010 | R/W | Slave address | 1~247 |
| 0x0011 | R/W | Baud index | 0~3 |
| 0x0012 | R/W | Parity | 0=None,1=Even,2=Odd |

# 9. Initial Setting Value

| Item | Initial Value |
| --- | --- |
| MODE | NORMAL |
| FREQ CH | CH01 / 400kHz |
| POWER | PWR1 / DEF 0.50W |
| LOW ALARM | 0.10W |
| HIGH ALARM | 5.00W |
| RS-485 | ADDR1 / 9600bps / NONE-8-1 |

# 10. Setting Storage

Operating mode, FREQ channel/frequency/BIT, POWER channel/LOW/HIGH/DEF and RS-485 settings saved by SET are stored in internal FLASH and retained after power-off.

# 11. Troubleshooting

| Symptom | Possible Cause | Action |
| --- | --- | --- |
| No output | STOP/alarm/SENSOR open | Check LCD, alarm and SENSOR |
| Err7 at power ON | SENSOR OPEN | Power OFF and check pin 13-25 SHORT |
| REMOTE does not work | Pin 9-22 contact or mode setting error | Check REMOTE wiring and mode |
| No RS-485 response | A/B, address, baudrate or parity mismatch | Check EXT:RS485 setting |

# 12. Maintenance

Daily: check cables/connectors, LCD/LEDs and alarm history

Weekly: check output repeatability, REMOTE contact and RS-485 response

Monthly: check transducer/cable insulation, alarm thresholds and setting backup

# 13. A/S

When requesting service, provide model name, serial number, alarm code, operating condition, installation photos and reproduction steps.

# 14. Revision History

| Version | Date | Description |
| --- | --- | --- |
| v4.0-EN | 2026-05-19 | Full LCD panel user manual with 25-pin D-SUB, actual panel image, detailed menu settings, REMOTE/EXT operation and bilingual output |